Project WoogleNet: Network Infrastructure Documentation

This repository contains the architectural design, hardware inventory, and deployment walkthrough for the WoogleNet home environment.

🌐 Network Architecture

Current State

- Primary Subnet: 10.0.0.1 / 255.255.0.0

- Wireless: Two isolated SSIDs (WoogleNet and `WoogleNetDMZ**

Future State (Target Configuration)

To achieve the goal of three distinct but communicating production networks, the environment will move to a VLAN-based architecture.

1. Production Environment (10.0.x.x)

Communication is permitted between these three VLANs.

| Network | VLAN | IP Range | Purpose | Assignment |
| --- | --- | --- | --- | --- |
| Network 1 | 10 | 10.0.1.1 - 10.0.1.10 | Management/HW | Static (Router, Switches, APs) |
| Network 2 | 2** | 10.0.2.10 - 10.0.2.99 | Servers/Services | Static (TrueNAS) / Mixed (VMs/Containers) |
| Network 3 | 30 | 10.0.3.100 - 10.0.3.254 | Personal Devices | DHCP (Phones, Laptops, Desktops) |

- Wireless Access: WoogleNet (2.5GHz & 5GHz) →Maps to VLAN 30.

2. IoT Environment (192.168.1.x)

- Subnet: 192.168.1.1/24

- Access: DHCP.

- Security: Internet-only; no routing permitted to Production, Guest, or Management networks.

- Wireless Access: WoogleNetIOT.

3. Guest/DMZ Environment (192.168.2.x)

- Subnet: 192.168.2.1/24

- Access: DHCP.

- Security: Internet-only.

- Wireless Access: WoogleNetGuest.

🛠 Hardware Inventory

Network Gear

| Device | Model | Quantity | Role |
| --- | --- | --- | --- |
| Gateway | UBNT Cloud Gateway Fiber (UCG-Fiber) | 1 | Primary Router/GW** |
| Switch | UBNT 24 Port | 2 | Core/Distribution |
| Switch | TPLINK 24 Port | 2 | Access/Edge |
| AP | UAC-LR | 3 | General Coverage (Attic, Basement, Living Room) |
| AP | UAC-Pro | 3 | High Density (Office, Game Room, TBD) |
| AP | UAC External | 1 | Outdoor (Backyard) |

Compute Gear

| Hardware | OS | Quantity | Role |
| --- | --- | --- | --- |
| Dell R72** | TrueNAS | 4 | Hypervisor / Storage Nodes |

📋 Software & Service Stack

OS & Virtualization

- Host OS: TrueNAS

- Container Host: Ubuntu Server is utilized as a dedicated VM host to run Docker and Docker Compose for maximum flexibility and compatibility.

Containers

Deployed across the TrueNAS cluster or Ubuntu VMs using the naming convention [Server][App].

- Media: Plex, TDARR, TMM

- Knowledge/Docs: Wiki

- Personal/Utility: Calibre, Immich, Vaultwarden, Tatuli

Virtual Machines

- DL: Download Manager

- AD: Active Directory / Domain Controller

- WEB: Web Server

- PROXY: Reverse Proxy (Entry point for external access)

📦 Docker on Ubuntu Setup

Manual Installation Steps

- Update the existing system packages.

- Install prerequisite packages to allow apt to use a repository over HTTPS.

- Add the official Docker GPG key for package verification.

- Set up the stable Docker repository.

- Install Docker Engine, CLI, and Docker Compose.

- Add the current user to the docker group to avoid using sudo for every command.

Automation Script

Save the following as setup-docker.sh, make it executable with chmod +x setup-docker.sh, and run it as root.

bash

#!/bin/bash

# WoogleNet Docker Installation Script for Ubuntu# Run as root or with sudo

set -e

echo "Updating system packages..."apt-get update && apt-get upgrade -y

echo "Installing prerequisites..."apt-get install -y ca-certificates curl gnupg lsb-release

echo "Adding Docker GPG key..."install -m 0755 -d /etc/apt/keyringscurl -fsSL https://download.docker.com/linux/ubuntu/gpg | gpg --dearmor -o /etc/apt/keyrings/docker.gpgchmod a+r /etc/apt/keyrings/docker.gpg

echo "Setting up Docker repository..."echo \  "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.gpg] https://download.docker.com/linux/ubuntu \  $(. /etc/os-release && echo "$VERSION_CODENAME") stable" | \  tee /etc/apt/sources.list.d/docker.list > /dev/null

echo "Installing Docker Engine and Compose..."apt-get updateapt-get install -y docker-ce docker-ce-cli containerd.io docker-buildx-plugin docker-compose-plugin

echo "Configuring permissions for current user..."# Gets the user who called sudoSUDO_USER=${SUDO_USER:-$(whoami)}usermod -aG docker $SUDO_USER

echo "Installation complete. Please log out and back in for group changes to take effect."docker --versiondocker compose version

🏷 Naming Conventions

Strict adherence to these conventions is required for DNS and management.

| Category | Convention | Example |
| --- | --- | --- |
| Servers | 3-Letter Uppercase | DAX, MAX, JAX, PAX |
| Containers | [Server Name][App] | DAXPlex, MAXImmich |
| Net Hardware | WoogleNet[Function][ID/Model] | WoogleNetRouter, WoogleNetSwitch01 |
| Access Points | [Location] | UPSTAIRS, DOWNSTAIRS, MAIN** |
| Personal | [Owner]_[Device]_[Model] | Eric_Phone_Pixel9, Eric_Desktop |
| IoT | [Location]_[Function]_[Num] | Office_Light_1 |

🗄 Rack Layout (42U)

| Unit | Component | Name/Details |
| --- | --- | --- |
| U1 | Router/GW | WoogleNetRouter_R01** |
| U2 | Patch Panel | — |
| U3 | Switch | WoogleNetSwitch01** |
| U4 | Patch Panel | — |
| U5 | Switch | WoogleNetSwitch02** |
| U6 | Patch Panel | — |
| U7 | Switch | WoogleNetSwitch03** |
| U8 | Patch Panel | — |
| U12 | Monitor | System Console** |
| U16 | Shelf | Keyboard / Mouse |
| U21 | Shelf | Accessories |
| U23 | KVM | Management Console** |
| U24-25 | Server | DAX |
| U26-27 | Server | MAX** |
| U28-29 | Server | JAX |
| U30-31 | Server | `PAX** |
| U32-39 | UPS | Power Redundancy Array** |

⚙️ DNS Configuration

Local DNS

Handled by UBNT Gateway for internal routing and local hostname resolution.

External DNS

Recommended settings based on latency and privacy:

| Provider | Primary | Secondary | Notes |
| --- | --- | --- | --- |
| Cloudflare | 1.1.1.1 | 1.0.0.1 | Fastest globally, high privacy |
| Google | 8.8.8.8 | 8.8.4.4 | Extremely robust infrastructure |
| Quad9 | 9.9.9.9 | 149.112.112.112 | Security focused (blocks malicious) |
| AdGuard | 94.140.14.14 | 94.140.15.15 | DNS-level ad blocking |

🚀 Setup Walkthrough

Phase 1: Gateway & VLAN Core

- Provision UCG-Fiber: Set up the primary gateway and configure the WAN interface.

- Define VLANs:

  - Create VLAN 10 (Mgmt), 20 (Servers), 30 (Personal).

  - Create separate subnets for IoT (192.168.1.1) and Guest (192.168.2.1).

- Firewall Rules:

  - Allow inter-VLAN routing for VLANs 10, 20, and 30.

  - Implement "Drop" rules for IoT and Guest VLANs to prevent access to any internal 10.0.x.x addresses.

Phase 2: Switching & Wireless

- Switch Adoption: Adopt all UBNT and TPLINK switches.

- Port Assignment: Assign ports to specific VLANs based on the connected device (e.g., Server ports →VLAN 20).

- Wireless SSID Mapping:

  - WoogleNet →VLAN 30

  - WoogleNetIOT →IoT Subnet

  - WoogleNetGuest →Guest Subnet

- AP Placement: Deploy APs to designated locations (Attic, Basement, Living Room, Office, Game Room, Backyard**

Phase 3: Compute & Services

- Hardware Install: Mount Dell R720s in U24-U31.

- TrueNAS Deploy: Install TrueNAS on all 4 nodes. Assign static IPs within the 10.0.2.x range.

- Ubuntu Host Setup:

  - Provision Ubuntu VMs on TrueNAS nodes.

  - Run the setup-docker.sh script to prepare container environments.

- Service Deployment:

  - Deploy VMs (AD, Proxy, Web, DL).

  - Deploy Containers according to the naming convention (e.g., DAXPlex).

- Verification: Ensure the Proxy VM can route traffic to the containerized services.

Phase 4: Client Onboarding

- Personal Devices: Connect devices to WoogleNet and verify DHCP assignment in the 10.0.3.x range.

- IoT Devices: Connect to WoogleNetIOT and verify they cannot ping the TrueNAS nodes.

- Guest Access: Verify WoogleNetGuest provides internet access only.**